Observatory Control System:  26/5/16

Arduino controller:

Serial Commands, strings, terminated by #

OPEN

Opens the roof – but only if the )mount is in safe position OR the mount sensor is disabled) AND it is (not raining OR the rain sensor is disabled)

CLOSE

Closes the roof – but only if the mount is in a safe position OR the mount sensor is disabled

INIT

Initializes system including initial EEPROM settings for sensors and resets rain sensor.

ABORT

Stops roof motion

FORCEOPEN

Opens the roof- regardless of sensors

FORCECLOSE

Closes the roof, regardless of sensors

NORAINSENSE

Disables the rain sensor

NOPARKSENSE

Disables the mount at park sensor

RAINSENSE

Enables the rain sensor

PARKSENSE

Enables the mount at park sensor

Broadcast Format – every 2 seconds:

$R,S,P,LL.L,C,ww.w#

- R – Boolean, rain, true=raining

- S - Enumerated, roofstatus, ASCOM standard

- P - Boolean, park, true=parked

- LL.L- light level decimal  (for expansion)

- C-cloud level 0-9 (for expansion)

- ww.w - Wind level km/h (for expansion)

- $ and # are delimiters

Alarms:

- If roof does not close within 1 minute of the onset of rain, alarm is sounded. Stops when roof is closed.

- If roof is closed, alarm sounds immediately if mount moves off park position.

ASCOM driver

Driver implements the following ASCOM commands:

- ShutterStatus

- Abort

- OpenShutter

- CloseShutter

Non-standard ASCOM properties automatically got from Arduino:

- Rain status

- Park status

Other commands use the generic ASCOM commands

- CommandBlind

- CommandBool

- CommandString

Serial communications uses 9,600 baud, no parity, 8-bit

Observatory Controller

Commands:

- Open : open safe – if mount parked, otherwise prompts if mount is not connected

- Close : close safe- if mount is parked, otherwise prompts if mount is not connected

- Que Open : wait for good conditions (dry, low humidty, clear) open roof and home mount

- Que Close: poor conditions (rain, humidity, cloud) causes mount to park and roof to close

- Park: park mount is roof is open

- Unpark: unpark mount is roof is open

- Home: if roof open, home mount

- Tracking off: turn mount tracking off

- Connect All: connect all devices

- Disconnect All: disconnect all devices

- Hibernate: open roof if closed, home and park mount, close roof

- Abort: stop mount slewing, tracking and any roof movement

- Individual setup, connect and disconnect for each device: (Chooser)

- Reset Roof: initialize roof (which resets potential in-between status and rain status

- Overrides – Force Open/Close: open and close, regardless of sensor status

- Overrides – Force Park: park mount, regardless of roof status

- Overrides – Force Home : home mount, regardless of roof status

- Enable / Disable Rain and Park sensors

Automatic functions:

- If roof is closed and mount is not parked or slewing, issues Abort command

- Updates roof position, mount status, rain, environment sensors every few seconds.

- Log window, for key messages to help with diagnostics.